| Component | Pin Function | ESP32 GPIO | Pin Type | Hardware Action / Notes |
| --- | --- | --- | --- | --- |
| All ToF Sensors | I2C SDA | GPIO 21 | Safe | Connect all 3 sensor SDA pins here. |
| All ToF Sensors | I2C SCL | GPIO 22 | Safe | Connect all 3 sensor SCL pins here. |
| Left ToF | XSHUT | GPIO 13 | Safe | Pull HIGH in code to activate. |
| Front ToF | XSHUT | GPIO 14 | Safe | Pull HIGH in code to activate. |
| Right ToF | XSHUT | GPIO 27 | Safe | Pull HIGH in code to activate. |
| Servo 1 | PWM Signal | GPIO 25 | Safe | Assign to ESP32 PWM Channel in code. |
| Servo 2 | PWM Signal | GPIO 26 | Safe | Assign to ESP32 PWM Channel in code. |
| Servo 3 | PWM Signal | GPIO 32 | Safe | Assign to ESP32 PWM Channel in code. |
| Servo 4 | PWM Signal | GPIO 18 | Safe | Assign to ESP32 PWM Channel in code. |
| Servo 5 | PWM Signal | GPIO 2 | Strapping | ⚠️ Must add a 10kΩ Pull-Down resistor to GND. |
| DC Motor 1 | Speed (PWM) | GPIO 33 | Safe | Connect to Motor Driver Enable pin. |
| DC Motor 1 | Direction (DIR) | GPIO 19 | Safe | Connect to IN1 (and NOT gate to IN2). |
| DC Motor 2 | Speed (PWM) | GPIO 4 | Safe | Connect to Motor Driver Enable pin. |
| DC Motor 2 | Direction (DIR) | GPIO 23 | Safe | Connect to IN1 (and NOT gate to IN2). |
| DC Motor 3 | Speed (PWM) | GPIO 16 | Safe | Connect to Motor Driver Enable pin. |
| DC Motor 3 | Direction (DIR) | GPIO 5 | Strapping | ⚠️ Must add a 10kΩ Pull-Up resistor to 3.3V. |
| DC Motor 4 | Speed (PWM) | GPIO 17 | Safe | Connect to Motor Driver Enable pin. |
| DC Motor 4 | Direction (DIR) | GPIO 15 | Strapping | ⚠️ Must add a 10kΩ Pull-Up resistor to 3.3V. |